Professional Audio/Visual Over Fiber Solution Manufacturer

4K Single LC Fiber Extender with Pigtail: HDMI to HDMI

OCTOPUS-HH-P01

- OCTOPUS-HH-P01 is a full 4k UHD video fiber extender pair, which resolution is up to 4096*2016@ 60Hz 4:4:4 (DSC2:1). The extender can transmit the HDMI2.0 signal with 18Gbps over one Single-mode or multi-mode fiber. The transmission distance is up to 300M over MMF and 20KM over SMF.

- The extender pair is using CWDM technology, there are 2 wavelength lasers in the extender pair, 1270nm and 1330nm.

Features

- Support 18Gbps video bandwidth.

- Video resolution up to 3840*2160 50/60HZ(YUV 4:4:4).

- Compliant with HDCP 2.3/2.2/1.X.

- HDR10，HDR10+，Dolby vision pass through.

- Support LPCM 7.1CH, Dolby 5.1, Dolby TrueHD, Dolby Atmos, DTS-HD Master, DTS：X Audio.

- EDID management through USB Type-C(female), support EDID pass-through and auto-adaptation. Built-in fixed list of commonly used EDIDs, allowing reading, displaying, and saving the EDID of the display end, and also support direct writing of .bin files.

- Transmission distance up to 300 meters over OM3 Multi-mode fiber, or 20KM over single-mode fiber.

- Support DDC/CEC/HPD.

- Compact size and detachable structure, easy and flexible for installation.

- TX can powered by 5V/1A or HDMI cable.

- Support mixed matching and interconnection with other interfaces (DVI, DP,USB-C) in the same product series.

Optical

- Optical source: CWDM DFB Laser

- Fiber: 1 core LC, Single Mode or Multi Mode

- Distance:  MM:300M  SM:20KM

Package

- TX x 1

- RX x 1

- 5V/1A power adapter x 2

- USB-A to USB-C cable x2

www.optav-tech.com

+86.137.2421.5827

freya@optav-tech.com

5th floor, High-tech industrial park, NO.1 Dezheng Road,

Shilong Community, Bao'an District, Shenzhen, China

5th floor, High-tech industrial park, NO.1 Dezheng Road,

Shilong Community, Bao'an District, Shenzhen, China

5th floor, High-tech industrial park, NO.1 Dezheng Road,

Shilong Community, Bao'an District, Shenzhen,China

Application

Specification

|  | Transmitter | Receiver |
| --- | --- | --- |
| Compliance | HDMI 2.0 |  |
| HDCP | HDCP 2.2/1.4 |  |
| Video Bandwidth | 18Gbps |  |
| Distance | Single-mode fiber: 20KM Multi-mode fiber: 300M |  |
| Supporting Resolution | 4Kx2K@60HZ(4:4:4 8bit) |  |
| Connector/Fiber | HDMI-A 19 Pin (male)/ LC | LC/HDMI-A 19 Pin (male) |
| CWDM filter | Integrated 1270nm/1310nm filter |  |
| Power Supply | 5V/1A or HDMI cable | 5V/1A |
| Power Consumption | 3W | 2W |
| Color | Black | Black |
| Dimension(LxWxH) | Pigtail: 190MM |  |
|  | Model: 64x35x12.3MM | Model: 64x35x12.3MM |
|  | Package: 250x180x37MM |  |
| Weight | Model: 85 g | Model: 85 g |
|  | Package：450 g |  |
| Environment | Operating Temperature Range: -10°C~60°C Storage Temperature Range: -40°C~85°C |  |
| Relative Humidity | 20-90% RH (no condensation) |  |

5th floor, High-tech industrial park, NO.1 Dezheng Road, 　　　　+86 137 2141 5827

Shilong Community, Bao'an District, Shenzhen,China

5th floor, High-tech industrial park, NO.1 Dezheng Road,

Shilong Community, Bao'an District, Shenzhen, China

+86.137.2421.5827

freya@optav-tech.com

www.optav-tech.com